[image: image7.png]


                             www.aww255.com

1210_L3210_L3250_L3251_L3260_L52

清零软件使用图解
清零前请取消打印任务，打印机用USB线接电脑并开启

双击[Resetter.exe]启动软件,
软件打开密码:  i-loadzone.com
点击[Select],选择Port:
[image: image1.jpg]) model = 13260 | www.i-loadzone.com

Model: L3260

Dest: Latin
Language: English
Port: Auto selection

www.i-loadzone.com

Sequential adjustment mode ‘

EPSON Adjustment Program

Select

You can select particular adjustment

Particular adjustment mode

Qui





打印机型号，然后点[OK]，如图。 
[image: image2.jpg]EPSON Adjustment Program
Model Name | [L3260 -l
Destination Latin |
Language English |
Port Auto selection -
ey | OK





 [如果port下拉列表中找不到你的打印机，请更换USB接口，并重新开打印机重试。]

2.点击[Particular adjustment mode]进入主菜单. 如图。
[image: image3.jpg]) model = 13260 | www.i-loadzone.com

Model: L3260

Dest: Latin
Language: English
Port: Auto selection

www.i-loadzone.com

EPSON Adjustment Program

Select

You can select particular adjustment

Sequential adjustment mode ‘ Parti

lar adjustment mode

Qui





3.选择[Waste ink pad counter]然后点[OK]. 如图。
[image: image4.png]model = L4268 | port = USB002 (L4260 Series) | AdjProg Ver.1.0.0

fusted items

Select an adjustment item from the below window and double-click to
execute it.

PF/EJ adjustment B
PF band adjustment

PW/First dot position adjustment

Head angular adjustment

Bi-D adjustment

PE adjustment

Scanner motor heat protection control L

-— Maintenance ——
Head cleaning
Ink charge

Waste ink pad counter
Shipping setting L

-— Appendix -—
Final check pattern print
EEPROM dump

Printer information check
Paper feed test

o

ok Setting Previous

- Selected items -

Model Name : L4268
ination : ECC

- Adjusted items -
Initial Setting :
Serial No. :

MAC Address :
Head ID :

PF/EJ adjustment
Smap :

Eject:

PF band adjustment
Black :

PW/First dot position
ladjustment





4.点击所有方框打上[V]，然后点击[Initialize],接着点[确定] 如图。

[image: image5.png]Adjustment order

model = L4268 | port = USB002 (L4260 Series) | AdjProg Ver.1.0.0

Adjusted items

Waste ink pad counter

NOTE : You have to replace waste ink pad if you initialize the protection
counter,

1) Click/[Check] button to check current protection counter's value.

2) Refflace waste ink pad with new one.

Get Status| Cleaning

- Selected items -
Model Name : L4268
Destination : ECC
Port : USB002 (L4260
Series)

- Adjusted items -
Initial Setting :
Serial No. :

MAC Address :

Head ID :

PF/EJ adjustment
Smap

Eject

PF band adjustment
Black :

PW/First dot position
ladjustment
Top :





[如果清零过程中报错，请取消打印任务并重开打印机再清，如果还不行请重启电脑再试]

5.当提示[plese turn off the printer] 时，请先关闭打印机，然后点击[确定],接着点[确定]重开打印机。如图。
[image: image6.png]Adjustment order

i model = 14268 | port = USBOO2 (L4260 Series) | AdjProg Ver.1.0.0

hdjusted items

Waste ink pad counter

NOTE : You have to replace waste ink pad if you ini
counter.

1) Click [Check] button to check current pr
2) Replace waste ink pad with new one.

Check &

counter's value.

™ Main pad cout

Please turn off the printer.
I™ Platen pad col

Check the current counter value. —>

nitialize the selected counters. —>

ish Cancel

Get Status| Cleaning

ize the protection

Check

Initialize

- Selected items -

Model Name : L4268
Destination : ECC
Port : USB002 (L4260
Series)

- Adjusted items -

Initial Settin

Serial No. :
MAC Address :
Head ID :

PF/EJ adjustment
Smap :
Eject:

PF band adjustment
Black :

PW/First dot position
ladjustment

Top :

Left:

Right :





亲，恭喜您，清零成功了！
www.aww255.com  www.aww255.net   米爸爸分享网

[image: image7.png]